NMIS/L-0912 SBC

Loading programs into FLASH-ROM

I had been having spotty success loading programs into the FLASH-ROM on the NMIS/L-0912 SBC. The programs loaded without apparent problems but, later, I discovered that some of the critical segments of the code were still in RAM, not in FLASH. Attempting to load some code into FLASH resulted in hangups that required erasing and reloading FLASH.

To figure out how to surmount these problems, I wrote a series of simple code fragments that progressively added new features to move code to FLASH. Along the way, I discovered that the download pacing was as important as the code itself -- particularly when loading a re-definition of a code word (such as ";"). The error message (xxx is not unique) could cause the next line to be mis-read, resulting in the desired word not being defined and all of the code ending up in the wrong place.

TABLES

My code uses a lot of tables of constants. These were not being put into FLASH in my previous attempts. The suggestion to re-define "," to include FLWORD didn't work for me. I found this approach to work, however:

CREATE TABLE			( normal definition

FLWORD				( move TABLE to FLASH

HERE					( will need this location to move data

xxxx ,  yyyy ,  zzzz ,  etc	( load data into RAM

FDP @ #BYTES FLMOVE		( move #BYTES from RAM to FLASH

( #BYTES must be a literal number, eg 6

( FLMOVE moves #BYTES from HERE to FDP

FDP @ #BYTES + FDP !          ( adjust FLASH DP

This results in a FLASH image that looks exactly like the RAM image from a CREATE sequence:

B8 6A xx xx yy yy zz zz  etc

If you fail to adjust the FLASH DP (dictionary pointer), bad things will happen; FLMOVE doesn't adjust FDP so the next attempt to move code to FLASH will encounter bytes that are already written -- and crash.

CONSTANTS & VARIABLES

I used the re-definitions suggested by Rob Chapman and they worked as advertised. In particular,

: CONSTANT

CONSTANT FLWORD

;

: VARIABLE

HERE CONSTANT 2 ALLOT

;

where the order is important. This results in the same code image in FLASH as the equivalent commands produce in RAM,:

0056 CONSTANT PORTP	produces 	B8 64 00 56 84 44

VARIABLE NPINGS		produces	B8 64 10 02 84 42,

where $0056 is the address of the data register for Port-P and $1002 is the RAM address where the value of the variable NPINGS is actually stored.

Re-writing the definitions helps with the error message, which occurs as soon as the : NAME is interpreted. Using a slow download pacing helps enormously, as well, since the re-definition causes an error message to display.

RAM BUFFERS

To construct a buffer in RAM with the address in FLASH, I did the following:

HERE CONSTANT BUFF		( define BUFF using the CONSTANT

( definition from above

100 ALLOT				( allot RAM space by moving the DP

ASSEMBLY CODE

I use a lot of assembly-language routines in my code. Putting these in FLASH is just a matter of adding FLWORD at the end of each code segment. I.e.,

CODE-SUB FLCODE				( define a code segment

A7A7 , A7A7 , A7A7 , 3D C,		( load bytes into RAM

END-CODE					( end the code definition

FLWORD					( move entire word to FLASH

This results in a FLASH image that looks just like the RAM image (before FLWORD) of

B8 70 A7 A7 A7 A7 A7 A7 3D

FORTH WORDS

The suggested way to move FORTH words to FLASH is to re-define the ';' word. I found this works if the word is re-written and the pacing is adjusted to allow for the error message. (There is a method for supressing error messages on the Tech site at New Micros. I found it worked -- once -- but subsequent re-definitions gave error messages. I ended up not using it. Besides, I had no idea what it was doing!)

: ;

[COMPILE] ; FLWORD

; IMMEDIATE FLWORD

Using this re-definition, I wrote a simple word:

: MAIN

BEGIN ?TERMINAL UNTIL KEY DROP

;

The resulting FLASH image was identical to the RAM image obtained with normal loading,

B8 36 C4 C9 B9 97 FF FC BB CD B8 A1 B8 41

USING EXTERNAL RAM

The external RAM on the NMIS/L-0912 adds about 28K of storage that I use heavily in my programs. This RAM is not visible to the CPU until the FORTH word EXRAM has been executed. I generally download a program called FFLASH.4th into EPROM to run upon startup that enables the external RAM for use by subsequent programs. The code for this program looks like:

( FFLASH.4Th - loads an INIT program

( to enable EXT RAM and set baud rate to 19200.

( March 2005 / cfg

cold

hex

: enableram

001a 00c0 !          	( set baud to 19200

exram				( enable external RAM

;

eeword					( move code to EPROM

a55a ffe ee!				( install autostart flag

' enableram cfa ffc ee!			( and execution address

This program will install an autostart flag at $0FFE and the address of the code at $0FFC. The enableram code itself usually loads at $0D0E. This code runs at startup (or reset), enabling external RAM (and, in this example, also setting the baud rate to 19.2 kbaud). It then falls through to the operating system.

AUTOSTARTING & REGAINING CONTROL

It is a simple matter to install an autostart flag to cause the program stored in FLASH to run automatically upon startup or reset. There are a variety of places one could place this flag, and two choices of flag are available. I tend to use the $A44A flag instead of the $A55A flag. The difference between these flags is that a program started with the $A44A flag can fall through into FORTH if it wishes whereas a program started with $A55A cannot. I like to have a way to recover from problems and getting into FORTH is a reasonable way to do this.

The following code comprises the main word of one of my programs. Inside the main loop is a word that looks for input on the serial port (?terminal); if a character is present, it executes a routine called dokey. One of the possible command characters that can be interpreted by dokey is CTRL-K. When this command is received, dokey calls a routine that sets the flag, killme, to TRUE (the routine init sets killme to FALSE). A true value for killme causes the begin - until loop to exit. At this point, the operator can choose to erase the autostart flag in preparation for reloading a program or not. In either case, the program will then halt and return control to FORTH.

: main

init

true mode !

begin                    ( start an infinite loop

mode @                 ( RUN mode?

if

pingcycle            ( yes, so take data

scale                ( scale by gains

output               ( send results

setgains             ( adjust gains for next cycle

then

5000 0 do i drop loop

?terminal              ( keypress?

if

dokey                ( yes, execute cmd

then

killme @               ( fetch exit flag, normally false

until

." erase autorun vector y/n? " key dup . cr

89 = if norun runvec c! then

;

(  w2flash.4th - WHAPS II code for FLASH RAM - 04 Feb 2005

hex

a44a d00 ee!               ( set auto-run flag in EEPROM

' main cfa d02 ee!

decimal

If the autostart flag used was $A55A, then it will not be possible to erase flash solely from the host computer. You will have to open up the unit and make some connections to the CPU card itself. See the section WRESTING CONTROL FROM THE CPU.

RE-LOADING A PROGRAM

It is possible to erase and reload a program from the host computer if a means is provided to exit to Forth such as described above. The procedure is as follows:

First, exit the program to Forth. Make sure the autostart flag byte is overwritten.

Re-read the sentence above. Make VERY sure the autostart flag is overwritten! Check this by dumping a region at $0D00:

HEX

D00 10 DUMP

and make sure the first two bytes are $FFFF. If they aren't, do it manually:

FFFF D00 EE!

and dump this region again to check.

Next, type FLASH and press <RETURN>. If you have changed the baud rate to something other than 9600 baud, change the baud rate of the host computer terminal program to 9600. Press <RETURN> again and you should see

Erase(E) or Program(P):

Type an "E".  The CPU monitor program will block-erase the available FLASH-RAM and display this message again. This time, respond with "P".

Before you can download new operating code, you must first download the file F12V50L.S19 (the Forth system) to the processor. This download must be done with timed-pacing.

When the download is complete, turn off power to the system (or short the reset pins). When you turn power back on, the code loaded into EEPROM (to enable external RAM and, in the case of the example, to set the baud rate to 19200) will execute and the processor will then drop into Forth. You should see the Forth prompt shortly after power-up. (Remember to adjust the baud rate as needed!)

The operating programs I have written for this processor consist of text files with the following code near the top:

: flblank ( - a)    ( find starting point if FLASH

f800 f800 8000 do

ff I 10 over + swap do

I c@ and

loop

ff = if drop I leave then

100

+loop

dup f800 = if cr ." Flash is full" cr else dup u. then

;

flblank fdp ! forget flblank     	( set the FDP

This code, from New Micros Inc., searches the FLASH-RAM for un-programmed space and sets the flash dictionary pointer, fdp, to the first empty location on $100 boundaries.

The body of my code contains re-defined words as described above and the code required to accomplish the functions of the code. The last word of my code is always MAIN:

hex

: main

init

begin                    ( start an infinite loop

?terminal              ( keypress?

if

dokey                ( yes, execute cmd

then

killme @               ( check exit flag

until

;

(  BSFLASH.4th - 96 freq code - 28 Aug 2005

a44a d00 ee!               ( set auto-run flag in EEPROM

' main cfa d02 ee!

decimal

The last few lines install the autostart flag, $A44A, and the address of MAIN into the autostart location in EEPROM.

When the code is loaded, either power can be cycled or you can type "COLD" to cause a reset to occur. This will force the initialization code (if installed) to run, followed by a call to MAIN.  The operating code will continue to run until power is removed or the killme flag is set again. (Note in the example above that the autostart vector must be over-written manually when the killme flag is set.

RUNNING TEST CODE IN RAM

describe the mods I typically make to squeeze the data space and leave room for code.

describe the process of validating FORTH words bottom-up-wise

WRESTING CONTROL FROM THE CPU

describe using a jumper on PDLC0 to ground to get to erase/program menu, use of the RESET jumper and CTRL-G to, maybe, get to FORTH. Then set HEX and over-write the $A55A flag(s) to regain control after power-cycle.

describe using the BDM port and an external program to over-write EPROM.

66666666666666666666666666666666666666666666